- ※产品分类※：晶圆测试和包装 测量分选设备（在左侧分类里选择※必填项）

- ※产品名称※： E+H 测量分选设备

- ※英文名※：   E+H Measuring sorting system

- 属性设置：（请在左侧三项里选择）

- 上传缩略图：（请和对应的word文档放在一起单独建立一个文件夹）

尺寸要求131*193像素

- 上传文件：（请和对应的word文档放在一起单独建立一个文件夹）

MX 6012.doc

- 产品简介：（不超过200个字符）

E+H成立于1968年，1978年进入半导体行业，在1996年的半导体展会上，E+H是第一个展示12英寸测量设备的公司。

此设备可按客户要求定制测量：厚度，电阻率(须有厚度)，表面粗糙度，P/N型等及设置分选个数。测量机构均为无接触测量，设备接触工件部分均采用聚醚醚酮材质，支持匹配客户指定的硅片盒。测量数据稳定准确，操作简单。

E+H产品众多，功能各异，欢迎来电详询。

英文

E+H OHG is founded in 1968.In 1978,E+H invented its first semiconductor applications.E+H was the first company showing a 300mm tool at the Semicon in 1996!

The equipment can be made according to customer requirements，like thinkness;resistivity(THK necessary);P/N;sorting and so on.The measuring parts are all non-contact, and the material of parts touching the workpiece are PEEK.Support for matching customer’s cassette.The measurement data is accurate, and the operation is very easy.

E+H has many products with different functions ,please contact us。

- 功能特点：（见下例格式）

#### ◆特点

英文：

High precision metrology

Flexible product adaptation near customer

Best cost benefit ratio compared to competitors

Long term customer’s care and product support

中文：

高精度测量

根据客户需求灵活调整产品

与竞争对手相比，最高的性价比

长期稳定的产品售后支持和客户关怀

- 规格参数：（见下例格式）

中文：

E+H 测量分选设备

| 技术规格: |  |  |  |
| --- | --- | --- | --- |
| 晶圆直径 |  |  | 300mm |
| 厚度范围 |  |  | 600-900µm |
| 最大翘曲 |  |  | 100µm |
| 电阻率范围 |  |  | 0.001-200Ohm•cm |
| 类型检查范围 |  |  | 0.020-200Ohm•cm |
| 厚度测量 |  |  |  |
| 厚度绝对精度°) |  |  | +/-0.3µm |
| TTV精度(MX 6012） |  |  | +/-0.1µm |
| 精度（MX 6012） |  |  | 0.05µm（1 Sigma） |
| 电阻率测量 |  |  |  |
| 准确度°) |  | 0.001-80 Ω•cm | +/-1% “) |
|  |  | 80-200Ω•cm | +/-5% |
| 精度*) |  | 0.001-80Ω•cm | 0.2%（1 Sigma） |
|  |  | 80-200Ω•cm | 2%（1 Sigma） |
| 边缘排除 |  |  |  |
| 厚度 |  |  | 5 mm |
| 电阻率 |  |  | 10 mm |
| 吞吐量（1个点） |  |  | >150晶圆/小时 |
| 发射接受站点 |  |  | 12 |

°) 校准温度+/- 2 K，梯度< 1 K/h时，实际值偏差、重复测量平均值均在这些范围内

*) 传感器中心到晶圆边缘的距离

“) 和校准样片有关

英文：

E+H Measuring sorting system

| Technical Specifications: |  |  |  |
| --- | --- | --- | --- |
| Wafer diameter |  |  | 300 mm |
| Thickness range |  |  | 600 - 900 µm |
| Max. Warp |  |  | 100 µm |
| Resistivity range |  |  | 0.001 – 200 Ohm•cm |
| Type check range |  |  | 0.020 – 200 Ohm•cm |
| Thickness Measurement |  |  |  |
| Thickness absolute Accuracy °) |  |  | +/- 0.3 µm |
| TTV Accuracy (MX 6012) |  |  | +/- 0.1 µm |
| Precision (MX 6012) |  |  | 0.05 µm (1 sigma) |
| Resistivity Measurement |  |  |  |
| Accuracy °) |  | 0.001 - 80 Ohm•cm | +/- 1 % “) |
|  |  | 80 - 200 Ohm•cm | +/- 5 % |
| Precision *) |  | 0.001 - 80 Ohm•cm | 0.2 % (1 sigma) |
|  |  | 80 – 200 Ohm•cm | 2 % (1 sigma) |
| Edge Exclusion **) |  |  |  |
| Thickness |  |  | 5 mm |
| Resistivity |  |  | 10 mm |
| Throughput (1 Point) |  |  | > 150 Wafer/hour |
| Station |  |  | 12 |

°) Deviation of real value, average of repeated measurements are within these limits, at calibration temperature +/- 2 K, gradient < 1 K/h

*) Distance from sensor center to wafer edge

“) Depends on quality of calibration samples

- 产品应用：（见下例格式）

中文：

◆ 应用：

• 硅片厂

• 晶圆制造厂

英文：

◆ APPLICATIONS

• Wafer maker

• Chip maker